module controlpath (

input  wire [5:0] opcode,

input  wire [5:0] funct,        // 5-bit function field from instr[4:0]

output reg  [3:0] alu_control,  // 4-bit control to ALU wrapper

output reg        alu_src,      // 1 => use rt, 0 => use immediate

output reg        immSel,       // if 0 -> 16bit imm else 26bit jmp

output reg        wr_reg,       // write register file

output reg        reg_dst,       // 1 => rd (R-type), 0 => rt (I-type)

output reg        rdMem,        // Read from data memory

output reg        wrMem,         // Write to data memory

output reg        mToReg,       //  if 1 => Meomry send to register else alu_result

output reg  [2:0] brOp,         // which branch to choose in branch comparator

output reg        isCmov,

output reg        isCall,

output reg        updPc

);

// ALU control encodings (4-bit for wrapper)

localparam ALU_LUI  = 4'b0000;

localparam ALU_ADD  = 4'b0001;

localparam ALU_SUB  = 4'b0010;

localparam ALU_AND  = 4'b0011;

localparam ALU_OR   = 4'b0100;

localparam ALU_XOR  = 4'b0101;

localparam ALU_NOR  = 4'b0110;

localparam ALU_SL   = 4'b0111;

localparam ALU_SRL  = 4'b1000;

localparam ALU_SRA  = 4'b1001;

localparam ALU_SLT  = 4'b1010;

localparam ALU_SGT  = 4'b1011;

localparam ALU_NOT  = 4'b1100;

localparam ALU_INC  = 4'b1101;

localparam ALU_DEC  = 4'b1110;

localparam ALU_HAM  = 4'b1111;

// Opcode definitions

// R-type: 000000

// I-type: ADDI:000001, SUBI:000010, ANDI:000011, ORI:000100, XORI:000101

//         NOTI:001100, INCI:001101, DECI:001110, HAMI:001111, LUI:010000

always @(*) begin

// Default values

alu_control = ALU_ADD;

alu_src     = 1'b1;    // default to rt (R-type)

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc       = 1'b0;

brOp        = 3'b100;

isCmov      = 1'b0;

isCall      = 1'b0;

case (opcode)

6'b000000: begin // R-type instruction

reg_dst  = 1'b1;   // write to rd

wr_reg   = 1'b1;   // enable write

alu_src  = 1'b1;   // use rt as operand_b

updPc    = 1'b1;

brOp     = 3'b100;

case (funct)

6'b000001: alu_control = ALU_ADD;

6'b000010: alu_control = ALU_SUB;

6'b000011: alu_control = ALU_AND;

6'b000100: alu_control = ALU_OR;

6'b000101: alu_control = ALU_XOR;

6'b000110: alu_control = ALU_NOR;

6'b000111: alu_control = ALU_SL;

6'b001000: alu_control = ALU_SRL;

6'b001001: alu_control = ALU_SRA;

6'b001010: alu_control = ALU_SLT;

6'b001011: alu_control = ALU_SGT;

6'b001100: alu_control = ALU_NOT;

6'b001101: alu_control = ALU_INC;

6'b001110: alu_control = ALU_DEC;

6'b001111: alu_control = ALU_HAM;

default:  alu_control = ALU_ADD;

endcase

end

// I-type instructions

6'b000001: begin // ADDI

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;   // write to rt

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000010: begin // SUBI

alu_control = ALU_SUB;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000011: begin // ANDI

alu_control = ALU_AND;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000100: begin // ORI

alu_control = ALU_OR;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000101: begin // XORI

alu_control = ALU_XOR;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000110: begin // NORI

alu_control = ALU_NOR;

alu_src     = 1'b0;   // use immediate

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;   // write to rt

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b000111: begin // SLI

alu_control = ALU_SL;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001000: begin // SRLI

alu_control = ALU_SRL;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001001: begin // SRAI

alu_control = ALU_SRA;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001010: begin // SLTI

alu_control = ALU_SLT;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001011: begin // SGTI

alu_control = ALU_SGT;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001100: begin // NOTI

alu_control = ALU_NOT;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001101: begin // INCI

alu_control = ALU_INC;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001110: begin // DECI

alu_control = ALU_DEC;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b001111: begin // HAMI

alu_control = ALU_HAM;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b010000: begin // LUI

alu_control = ALU_LUI;

alu_src     = 1'b0;

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

// MEMORY Control Signals

6'b010001: begin //LD

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

immSel      = 1'b0;

wr_reg      = 1'b1;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b1;

wrMem       = 1'b0;

mToReg      = 1'b1;

brOp        = 3'b100;

updPc    = 1'b1;

end

6'b010010: begin //SW

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b1;

mToReg      = 1'b0;

brOp        = 3'b100;

updPc    = 1'b1;

end

// BRANCH

//immSel = 1 means jump

6'b100000: begin //BR

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

brOp        = 3'b000;

immSel      = 1'b1;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

end

6'b100001: begin //BMI

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

brOp        = 3'b001;

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

end

6'b100010: begin //BPL

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

brOp        = 3'b010;

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

$display("BPL Instruction is hit brop is %h",brOp);

end

6'b100011: begin //BZ

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

brOp        = 3'b011;

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

end

6'b010100: begin  //MOVE

alu_control = ALU_ADD;

alu_src     = 1'b0;

immSel      = 1'b0;  // send 0 in immediate

wr_reg      = 1'b1;

reg_dst     = 1'b0; //rt ( isa file has some wrong)

brOp        = 3'b100;

isCmov      = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

$display("Move is set ");

end

6'b010101: begin  //CMOV

alu_control = ALU_ADD;

alu_src     = 1'b1;

immSel      = 1'b0;  // send 0 in immediate

wr_reg      = 1'b1;

reg_dst     = 1'b1; //rd

brOp        = 3'b100;

isCmov      = 1'b1;

mToReg      = 1'b0;

updPc    = 1'b1;

$display("CMOV is set ");

end

//SYSCALLS

6'b100101: begin //NOP

// Same as default

alu_control = ALU_ADD;

alu_src     = 1'b0;

immSel      = 1'b1;  // send 0 in immediate 26 bit

wr_reg      = 1'b0;

reg_dst     = 1'b0; //rt ( isa file has some wrong)

brOp        = 3'b100;

isCmov      = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b1;

end

6'b100110: begin //CALL

// Same as default

alu_control = ALU_ADD;

alu_src     = 1'b0;

immSel      = 1'b0;  // send 0 in immediate 16 bit

wr_reg      = 1'b1;

reg_dst     = 1'b0; //rt

brOp        = 3'b100;

isCmov      = 1'b0;

mToReg      = 1'b0;

isCall      = 1'b1;

updPc    = 1'b1;

end

6'b100100: begin //HALT

alu_control = ALU_ADD;

alu_src     = 1'b0;   // use immediate

brOp        = 3'b100;

immSel      = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;   // write to rt

rdMem       = 1'b0;

wrMem       = 1'b0;

mToReg      = 1'b0;

updPc    = 1'b0;

$display("program halted");

end

default: begin

// No operation - keep defaults

alu_control = ALU_ADD;

brOp        = 3'b100;

alu_src     = 1'b0;

wr_reg      = 1'b0;

reg_dst     = 1'b0;

updPc    = 1'b0;

end

endcase

end

endmodule